Theoretical Documentation

DAD drift v1.0

By Mike Devin Fuchs

Table of Content

1.	Environmental Constants	4

1.1.	Dynamic Viscosity of Air	4

1.2.	Saturated Vapor Pressure of Water	4

1.3.	Wet-Bulb Temperature	4

1.4.	Diffusion Coefficient of Water in Air	4

1.5.	Density of Humid Air	4

1.6.	Density of Water	5

1.7.	Absolute humidity	5

1.8.	Relative humidity	5

1.9.	Schmidt Number	5

1.10.	Specific Latent Heat of Water	6

1.11.	Enthalpy of Vaporization of Water	6

2.	Micrometeorological Model	6

2.1.	Volume	6

2.2.	Mass	7

2.3.	Internal Energy	7

2.4.	Temperature	8

2.5.	Humidity	8

3.	Droplet Model	8

3.1.	Evaporative Flux of the Active Ingredient	8

3.2.	Solution Parameters	9

3.3.	Initial Droplet Parameters	10

3.4.	Droplet Kinetics	10

3.5.	Droplet Evaporation	11

3.6.	Initial Droplet velocity	12

3.7.	Cumulated Probability	13

4.	Dispersion Model	13

4.1.	Gaussian Diffusion Model	13

4.2.	Deposition Model	14

4.3.	Vertical Wind Profile	14

5.	Input/Output Documentation	15

5.1.	General Description	15

5.2.	Application Input File	16

5.3.	Environmental Input File	16

5.4.	Control Input File	17

5.5.	Drop Size Spectrum Input File	17

5.6.	Landscape Input File	17

6.	References	18

# Environmental Constants

## Dynamic Viscosity of Air

The dynamic viscosity of air (kg m-1s-1) was calculated according to Sutherland-law:

|  | µ=µ0*T0+ST+S*TT032 | 1 |
| --- | --- | --- |

where, µ0 (kg m-1s-1) is the reference viscosity for air [1.716*10-5 (kg m-1s-1)], T0 (K) is the reference temperature for air [273 (K)], and S (K) is the Sutherland constant for air [111 (K)] (Kim et al., 2004).

## Saturated Vapor Pressure of Water

The saturated vapor pressure of water (Pa) was calculated as follows:

|  | psat=p0*107.5*TT+237.3 | 2 |
| --- | --- | --- |

where, T (C°) is the temperature and p0 (Pa) is the vapor pressure of water at 0 °C [610.7 (Pa)] (Holterman, 2003).

## Wet-Bulb Temperature

The wet-bulb temperature (C°) is computed as follows:

|  | Tw=T*atan0.151977*Rh+8.31365912+atanT+Rh -atanRh-1.676331+0.00391838*Rh32*atan0.023101*Rh-4.686035 | 3 |
| --- | --- | --- |

where, T (C°) is the temperature and Rh (%) is the relative humidity (Stull, 2011).

## Diffusion Coefficient of Water in Air

The diffusion coefficient of water in air (m² s-1) was calculated as follows:

|  | DV=D0*(1+0.0071*T) | 4 |
| --- | --- | --- |

where, D0 (m² s-1) is the reference diffusion coefficient [21.2*10-6 (m² s-1)] and T (C°) is the temperature (Holterman, 2003).

## Density of Humid Air

The density of humid air (kg m-3) is calculated according to the ideal gas law:

|  | ρhumid air=pd*Md+Pv*MvR*T | 5 |
| --- | --- | --- |

where, pd (Pa) is the partial pressure of dry air, Md (kg mol-1) is the molar mass of dry air, pv (pa) is the pressure of water vapor, Mv (kg mol-1) is the molar mass of water vapor, R [J K-1mol-1] is the universal gas constant, and T (K) is the temperature.

## Density of Water

The density of water (kg m-³) is computed as follows:

|  | ρwater=999.85308+6.32693*10-2*T-8.523829*10-3*T2 +6.943248*10-5*T3-3.821216*10-7*T4 | 6 |
| --- | --- | --- |

where, T (°C) is the temperature (Jones and Harris, 1992).

## Absolute humidity

The absolute humidity (kg m-³) is calculated as follows:

|  | Ah=Rh*ρsatRh2o*T Ah=kgm*s²*s2*K*kgm2*kg*1K=kgm³ | 7 |
| --- | --- | --- |

where, Rh2o (J kg-1K-1) is the specific gas constant of water vapor, psat (pa) is the saturation vapor pressure, Rh (-) is the relative humidity, and T (K) is the temperature.

## Relative humidity

The relative humidity (-) is calculated as follows:

|  | Rh=mh2o*Rh2o*TV*ρsat Rh=kg*m2*kgs2*K*kg*K*1m3*m*s2kg=- | 8 |
| --- | --- | --- |

where, mh2o (kg) is the mass of water, Rh2o (J kg-1K-1] is the specific gas constant of water vapor, T (K) is the temperature, psat (pa) is the saturation vapor pressure, and V (m³) is the volume.

## Schmidt Number

The Schmidt number of water (-) is calculated as follows:

|  | Scwater=μairρair*DV Scwater=kgm*s*m3kg*sm2=[-] | 9 |
| --- | --- | --- |

where, µair (kg m-1s-1) is the dynamic viscosity of air, pair (Pa) is the density of air, and DV [m²/s] is the diffusion coefficient of water (Løfstrøm et al., 2013).

## Specific Latent Heat of Water

The specific latent heat of water (kJ kg-1) can be expressed as follows:

|  | λ=Hvh2oMh20 | 10 |
| --- | --- | --- |

where, Hvh20 (kJ mol-1) is the enthalpy of vaporization of water and Mh2o (g mol-1) is the molar mass of water (Brecht, 2019).

## Enthalpy of Vaporization of Water

The enthalpy of vaporization of water (kJ mol-1) can be estimated as follows:

|  | Hvh2o=50.09-0.9298*T1000-65.19*T10002 | 11 |
| --- | --- | --- |

where, T (K) is the water temperature (Brecht, 2019).

# Micrometeorological Model

## Volume

For the volume estimation of the microclimate the following assumptions where made:

- At the bottom the volume is limited by the ground.

- At the top the volume is limited by the boom height and vertical dispersion coefficient (σv).

- In both horizontal dimensions the volume is held constant. Assuming the mass exchanges between two neighboring units of application area being reflected of each other, cancelling out any mass changes.

Hence, the volume (m³) above one unit of application area is calculated as follows:

|  | Vt=bheight+σv | 12 |
| --- | --- | --- |

where, bheight (m) is the boom height and σv (m) vertical dispersion parameter.

The change of volume is calculated as follows:

|  | δVti=Vti-V(ti-1) | 13 |
| --- | --- | --- |

where, ti is the current time-step, ti-1 is the previous time-step, and V (m³) is the volume.

## Mass

The change in mass of water vapor (kg) is calculated as follows:

|  | δmh2o= δV*Ah | 14 |
| --- | --- | --- |

where, δV (m³) is the change in volume and Ah (kg m-³) is the absolute humidity of the global environment.

The change of air mass is calculated as follows:

|  | δmair= δV*ρhumid air-δmh2o | 15 |
| --- | --- | --- |

where, δV (m³) is the change in volume, δmh2o (kg) is the mass change of water vapor, and ρhumid_air (kg m-³) is the density of humid air.

The mass of air is calculated as follows:

|  | mairti=mairti-1+ δmair(ti) | 16 |
| --- | --- | --- |

where, mair(ti-1) (kg) is the mass of air in previous time-step and δmair(ti) (kg) is the change of air mass in the current time-step.

The mass of water is calculated as follows:

|  | mh2oti=mh2oti-1+ δmh2oti+δmh2o,evapti-1 | 17 |
| --- | --- | --- |

where, mh2o(ti-1) (kg) is the mass of water vapor of the previous time-step, δmh2o(ti) (kg) is the mass change of water vapor in the current time-step, and δmh2o,evap(ti-1) (kg) is the mass of evaporated water in the previous time-step.

## Internal Energy

The internal energy (J) of the system is described as follows:

|  | Et0=mh2ot0*cph2omairt0*cpair Eti=Eti-1+δmh2oti*cph2o+δmairti*cpair-Eevapti-1 | 18 |
| --- | --- | --- |

where, mh2o (kg) is the mass of water vapor, mair (kg) is the mass of air, cp_h2o (J kg-1) is the specific heat capacity of water, cp_air (J kg-1) is the specific heat capacity of air, E (J) is the internal energy of the system, Eevap (J) is the energy used during evaporation, and δEdrop (J) is the change in energy of the droplets caused by the change in temperature. With t0 indicating the first time-step, ti indicating the current time-step and ti-1 indicating the previous time-step.

The energy used during evaporation (J) is calculated as follows:

|  | Eevap=mh2o,evap*Hsh2o-mh2o,evap*Twb*cph2o | 19 |
| --- | --- | --- |

where, mh2o,evap (kg) is the mass of evaporated water, Hsh2o (J kg) is the specific latent heat of vaporization of water, Twb (K) is the wet-bulb temperature, and cp_h2o (J kg-1) is the specific heat capacity of water.

## Temperature

The temperature (K) of the volume is estimated as a function of the internal energy and the masses of water vapor and air:

|  | T=Emh2o*cph2o+mair*cpair | 20 |
| --- | --- | --- |

where, E (J) is the internal energy of the control volume, mH2O (kg) is the mass of water vapor, mair (kg) is the mass of air, cp, H2O (J kg-1) is the specific heat capacity of water, and cp,air (J kg-1) is the specific heat capacity of air.

## Humidity

The relative humidity (-) of the volume is estimated as follows:

|  | Rh=mH2ORH2OTVpsat(T) | 21 |
| --- | --- | --- |

where, mH2O (kg) is the mass of water vapor, RH2O (J kg-1K-1) is the specific gas constant of water, T (K) is the temperature, V (m³) is the volume, and psat(T) (Pa) is the saturated vapor pressure estimated by an empirical relation to temperature.

# Droplet Model

## Evaporative Flux of the Active Ingredient

The evaporative flux of the active ingredient (kg m-²s-1) is estimated based on the linear relationship to vapor pressure and molar mass:

|  | FAI=4.07*10-7*pAI*MAI | 22 |
| --- | --- | --- |

where, pAI (Pa) is the vapor pressure of the active ingredient and MAI (kg mol-1) is the molar mass of the active ingredient (Mackay and van Wesenbeeck, 2014).

## Solution Parameters

For the solution of AI in water, the amount of AI (kg) and the volume of water (L) is known. The volume of AI (m³) is calculated as follows:

|  | VAI=mAIρAI | 23 |
| --- | --- | --- |

where, mAI (kg) is the mass of AI and ρAI (kg m-³) is the density of the AI.

The mass of water (kg) was computed as follows:

|  | mwater=Vwater*ρwater | 24 |
| --- | --- | --- |

where, Vwater (m³) is the volume of water and ρwater (kg m-³) is the density of water.

The volumetric fraction (-) of AI was calculated as:

|  | xvolAI=VAIVAI+Vwater | 25 |
| --- | --- | --- |

where, Vwater (m³) is the volume of water and VAI (m³) is the volume of active ingredient.

The density of the droplet (kg m-³) was calculated as:

|  | ρdrop=mwater+mAIVwater+VAI | 26 |
| --- | --- | --- |

where, Vwater (m³) is the volume of water, VAI (m³) is the volume of AI, mAI (kg) is the mass of AI, and mwater (kg) is the mass of water.

The molar fraction of water (-) was calculated as follows:

|  | xmolwater=mwaterMwatermwaterMwater+mAIMAI | 27 |
| --- | --- | --- |

where, Mwater (kg mol-1) is the molar mass of water, MAI (kg mol-1) is the molar mass of AI, mAI (kg) is the mass of AI, and mwater (kg) is the mass of water.

## Initial Droplet Parameters

The initial droplet diameter (m) is known, and the initial volume (m³) is calculated as:

|  | Vini=π6*Dini³ | 28 |
| --- | --- | --- |

where, Dini (m) is the initial droplet diameter.

The initial mass of AI (kg) and the initial mass of water (kg) are then computed as:

|  | miniAI=xvolAI*Vini*ρAI=Vini*cAI miniwater=ρwater*Vini-miniAIρAI | 29 |
| --- | --- | --- |

where, Vini (m³) is the initial droplet volume, ρAI (kg m-³) is the density of the AI, ρwater (kg m-³) is the density of water, xvolAI (-) volumetric fraction of AI, and CAI (kg m-3) is concentration of AI in the spray solution.

The initial molar fraction of water (-) is equal to the molar fraction of water (-) calculated in equation 27.

## Droplet Kinetics

The momentum equation describes the deceleration (m s-²) as follows:

|  | δvδt=g-vdropτm | 30 |
| --- | --- | --- |

where, g (m s-²) is the earth-surface gravitational acceleration, vdrop (m s-1) is the droplet speed, and τm (s) is the relaxation time (Løfstrøm et al., 2013).

The relaxation time calculated as:

|  | τm=ρdrop*(Ddrop)²18*μair*f τm=kgm³*m2*m*skg=[s] | 31 |
| --- | --- | --- |

where, ρdrop (kg m-³) is the droplet density, Ddrop (m) is the droplet diameter, µair (kg m-1s-1) is the dynamic viscosity of air, and f (-) is a factor describing the deviation from the Stokes flow (Løfstrøm et al., 2013).

With f (-) being calculated as follows:

|  | f=1.0+0.15*Re0.687 | 32 |
| --- | --- | --- |

where, Re (-) is the Reynolds number (Løfstrøm et al., 2013).

The Reynolds number [-] is calculated as following:

|  | Re=Ddrop*vdrop*ρairμair Re=m*ms*kgm3*m*skg=[-] | 33 |
| --- | --- | --- |

where, Ddrop (m) is the droplet diameter, vdrop (m s-1) is the droplet velocity, ρair (kg m-³) is the density of air, and µair (kg m-1s-1) is the dynamic viscosity of air (Løfstrøm et al., 2013).

When setting the change in velocity to zero in equation 30 the settling velocity of the droplet can be described as:

|  | Vdrop=g*ρdrop*(Ddrop)²18*μair*f | 34 |
| --- | --- | --- |

where, Ddrop (m) is the droplet diameter, ρdrop (kg m-³) is the droplet density, g (m s-²) is the earth-surface gravitational acceleration, µair (kg m-1s-1) is the dynamic viscosity of air, and f (-) is a factor describing the deviation from the Stokes flow. This is similar to equation 4 of Holterman (2003) with the addition of the deviation from Stokes flow f (-).

## Droplet Evaporation

During the initial period of droplet lifetime, the evaporation rate is mainly driven by the cooling of the droplet, until it reaches the wet-bulb temperature. After this transient process the evaporation of the droplet can be approximated by steady evaporation. Since the transient process occurs on a time scale much smaller than the total lifetime of the droplet, it can be neglected (Holterman, 2003). The mass change (kg) of AI and water in the droplet is described as follows:

|  | δmAIδt=π*Ddrop2*FAI δmwaterδt=π*Ddrop2*Fwater | 35 |
| --- | --- | --- |

where, Ddrop (m) is the droplet diameter, FAI (kg m-²s-1) is the flux of AI through the boundary layer, and Fwater [kg/m²s] is the flux of water through the boundary layer.

The flux of AI (kg/m²s) was estimated by equation 22 and the flux of water [kg/m²s] is computed as follows:

|  | Fwater=Shwater*DVwater*MwaterDdrop*R*Twb*psat*Rh-psatwb*xmolwater Fwater=m²s*kgmol*1m*s2*mol*Kkg*m2*1K*kgm*s2=kgm2*s | 36 |
| --- | --- | --- |

where, Shwater (-) is the Sherwood number for water, DVwater (m² s-1) is the diffusion coefficient of water, Mwater (kg mol-1) is the molar mass of water, Ddrop (m) is the droplet diameter, R (kgm² s-²mol-1K-1) is the universal gas constant, Twb (K) is the wet-bulb temperature, psat (Pa) is the saturated vapor pressure of water at ambient temperature, Rh (-) is the relative humidity, psatwb (Pa) is the saturated vapor pressure of water at the wet-bulb temperature, and xmolwater (-) is the molar fraction of water in the droplet (Løfstrøm et al., 2013).

The Sherwood number of water (-) is calculated as:

|  | Shwater=1.0+0.276(Re)12*Scwater13 | 37 |
| --- | --- | --- |

where, Re (-) is the Reynolds number and Scwater (-) is the Schmidt number (Holterman, 2003; Løfstrøm et al., 2013).

## Initial Droplet velocity

The initial droplet velocity is generally assumed to be equal to the average liquid sheet velocity, with the liquid sheet velocity being equal to the liquid discharge velocity (m s-1) computed as:

|  | UL=2*PnρL12 UL=m3kg*kgm*s2=m2s2=ms | 38 |
| --- | --- | --- |

where, Pn (kg m-1s-²) is the spray pressure and ρL (kg m-³) is the density of the spray liquid (Dorr et al., 2013).

Adjusted for the nozzle angle the initial vertical droplet velocity (m s-1) can be described as:

|  | Vv=cosα*UL | 39 |
| --- | --- | --- |

where, UL (m s-1) is the liquid discharge velocity and α (°) is the nozzle angle.

## Cumulated Probability

The cumulated probability (-) of a normal distribution above a given value is calculated as follows:

|  | p(x)=1-121+erfx-μσ2 | 40 |
| --- | --- | --- |

where, x is the value, µ is the mean value and σ is the standard deviation.

# Dispersion Model

## Gaussian Diffusion Model

The Gaussian puff predicts the droplet concentration (kg m-3) at a given position (x,y,z) and as function of time as follows:

|  | Cx,y,z=M2π32σh2σvexp-12x-Utσh2exp-12yσh2exp-12z-zc(t)σv2 C=kgm3*mm*mm*mm=kgm3 | 41 |
| --- | --- | --- |

where, x, y and z (m) are the coordinates in x, y and z direction, respectively, t (s) is the time,  M (kg) is the mass release from the nozzle, σh and σv (m) are the dispersion coefficients in the horizontal and vertical directions, respectively, U (m s-1) is the wind speed, and zc (m) is the mean puff position in z direction as a function of time as predicted in the droplet model (Zannetti, 1990).

The vertical (σv) and horizontal (σh) dispersion coefficients are described by the following empirical relations to time, distance, and wind speed:

|  | σh=βσθ*x1+0.90.001*xU12 [σh]=β*ms12 = s12*ms12 = m | 42 |
| --- | --- | --- |
|  | σv=ασw*xu*1+0.90.02*xU12 [σv]=α*ss12 =ms12*ss12 = m | 43 |

where, t (s) is the time, x (m) is the distance, U (m s-1) is the wind speed, σw and σθ (-) are the unitless vertical and horizontal wind velocity standard deviation used as empirical calibration parameters, α (m s-1/2) is the empirical unit-correction factor for the vertical dispersion coefficient, and β (s1/2) is the empirical unit-correction factor for the horizontal dispersion coefficient (Draxler, 1976; Souto et al., 2001).

## Deposition Model

The deposition model predicts the deposition pattern emitted by one unit of application area. The droplet deposition rate (Fdep, kg m-2s-1) at a given position (x,y,z) and as function of time is estimated as follows:

|  | Fdep(x,y,z,t)=vdt*Cx,y,z,t | 44 |
| --- | --- | --- |

where, vd (m s-1) is the vertical droplet velocity as a function of time as predicted by the droplet model and C (kg m-3) is the concentration as a function of the position in three-dimensional space and time as predicted by the dispersion model (Lebeau et al., 2011; Løfstrøm et al., 2013).

## Vertical Wind Profile

Above the canopy the vertical wind profile is described by the logarithmic law. Within the canopy the vertical wind profile is described as an exponential function of the leaf area index.

|  | Uz=U*κ*lnz-dz0 if z≥Hc | 45 |
| --- | --- | --- |
|  | Uz=UH*expLAI2*zHc-1 if z<Hc | 46 |

where, U* (m s-1) is the friction velocity, κ (-) is the von Karman constant (0.4), z (m) is the height above ground, d (m) is the zero plane displacement, z0 (m) is the roughness length, Hc (m) is the canopy height, UH (m) is the wind speed at the top of the canopy as calculated by equation 45, and LAI (m² m-2) is the leaf area index (Sypka and Starzak, 2013). The roughness length is the height at which the wind speed becomes zero and can be approximated as one-tenth of the height of the surface roughness elements. The zero plane displacement can be assumed to be 0.63 times the canopy height (Butler Ellis and Miller, 2010).

# Input/Output Documentation

## General Description

The DAD drift model is comprised of multiple sub-models (Fehler! Verweisquelle konnte nicht gefunden werden.). First, the droplet model and the micrometeorological model are run in parallel for all defined droplet classes, with a droplet class representing droplets in a specific size range characterized by the mean droplet diameter. The droplet model predicts the motion and mass balance of droplets representative of their droplet class based on weather conditions (e.g., wind speed, wind direction, ambient pressure), droplet parameters (e.g., velocity, size, density), release height, and the micrometeorological model (i.e., humidity, temperature). The micrometeorological model accounts for indirect interactions between individual droplets within the spray cloud by estimating the change in humidity and temperature caused by droplet evaporation in the droplet model. Second, the dispersion model describes the atmospheric dispersion of a cloud of airborne droplets. The droplet trajectories provided by the droplet model are used to estimate the mean position of the three-dimensional puff and to predict the three-dimensional concentration field.

Figure 1: Schematic representation of the DAD drift model flow.

Third, the deposition model combines the vertical droplet velocity and the concentration field to predict the deposition mass flux at each deposition height, resulting in a two-dimensional deposition pattern. The dispersion model and the deposition model are executed sequentially for all droplet classes. Finally, the deposition pattern is used to predict either a drift deposition curve or a two-dimensional landscape-level drift deposition pattern as model output. The individual sub-models are described in the following sections, for additional details please refer to the theoretical model documentation.

## Application Input File

In the application input file “application_input.txt” a total of 14 parameters containing information relevant to the application of the active ingredient are defined. For the landscape mode 10 parameters are necessary, for the drift curve mode 12 parameters are necessary. Only one of the three parameters “app_rate_mh”, “app_rate_mha”, and “app_rate_kgha” must be defined for the model to run properly.

Table 1: application_input.txt input parameters.

| Parameter Name | Unit | Description |
| --- | --- | --- |
| tractor_speed | (m s-1) | Tractor speed, only used in combination with “app_rate_mh”. |
| boom_width | (m) | Width of the spray boom. |
| boom_height | (m) | Height of the spray boom above ground. |
| nozzle_angle | (°) | Deviation of nozzle angle from vertically downward spraying. |
| application_pres | (kPa) | Spray pressure used during application. |
| app_rate_mh | (m³ h-1) | Application rate in meters cubed per hour, no data value -99. |
| app_rate_mha | (m³ ha-1) | Application rate in meters cubes per acre, no data value -99. |
| app_rate_kgha | (kg ha-1) | Application rate in kilograms per acre, no data value -99. |
| sol_concentration | (kg m-³) | Concentration of the spray solution. |
| AI_density | (kg m-³) | Density of the active ingredient. |
| AI_molar_mass | (kg mol-1) | Molar mass of the active ingredient. |
| AI_vapor_pressure | (Pa) | Vapor pressure of the active ingredient. |
| swath_number | (-) | Number of spray swaths, only used in spray curve mode. |
| field_length | (m) | Length of the field (swath), only used in spray curve mode. |

## Environmental Input File

In the environmental input file “environment_input.txt” a total of 11 parameters containing information about the environmental conditions during application are defined. The following table gives an overview of the parameters:

Table 2: environment_input.txt input parameters.

| Parameter Name | Unit | Description |
| --- | --- | --- |
| temperature | (°C) | Ambient temperature. |
| humidity | (-) | Relative humidity. |
| wind_speed | (m s-1) | Wind speed. |
| wind_height | (m) | Height at which the wind speed was measured. |
| wind_direction | (°) | Wind direction relative to the spray path. |
| ambient_pressure | (kPa) | Ambient pressure. |
| sigma_horizontal | (m) | Horizontal wind standard deviation. |
| sigma_vertical | (m) | Vertical wind standard deviation. |
| roughness_height | (m) | Surface roughness height. |
| canopy_height | (m) | Canopy height. |
| LAI | (m² m-²) | Leaf area index. |

## Control Input File

In the control input file “control_input.txt” the main parameters of model execution (i.e. mode, deposition height, maximum distance, and number of landscape files) and additional input files are defined. The “dsd_file_name” defines which file contains the drop size distribution necessary for model execution and the “landscape_file_name” defines one or more raster input files defining areas of application in the landscape mode. The following table gives an overview of the parameters:

Table 3: control_input.txt input paramteres.

| Parameter Name | Unit | Description |
| --- | --- | --- |
| mode | (-) | Mode of model execution: 0 = landscape mode 1 = drift curve mode |
| dep_height | (m) | Height at which the deposition is calculated. |
| max_dist | (m) | Maximum distance used during drift pattern prediction. |
| field_count | (-) | Number of landscape files provided, only used in landscape mode. |
| dsd_file_name | (-) | Name of the file containing the drop size distribution. |
| landscape_file_name | (-) | Name(s) of the file(s) containing the landscape raster inputs, only used in landscape mode. |

## Drop Size Spectrum Input File

The drop size spectrum file, defined in the control input file, contains two columns. In the first column the mean droplet diameter (m) of each droplet size class is defined, and in the second column the cumulated fraction of exceedance (-) is defined.

## Landscape Input File

The landscape input files, defined in the control input file, are one or more raster files of the “.asc” file format. Containing the information on where application is happening. A raster cell where application is happening has a value of 1 assigned, all other cells should typically be assigned the no data value of -99.

# References

Brecht, B.M., 2019. Die urbane Wärmebelastung unter Einfluss lokaler Faktoren und zukünftiger Klimaänderungen. Wissenschaftliche Berichte des Instituts für Meteorologie und Klimaforschung des Karlsruher Instituts für Technologie. KIT Scientific Publishing. https://doi.org/10.5445/KSP/1000099242

Butler Ellis, M.C., Miller, P.C.H., 2010. The Silsoe Spray Drift Model: A model of spray drift for the assessment of non-target exposures to pesticides. Biosyst. Eng. 107, 169–177. https://doi.org/10.1016/j.biosystemseng.2010.09.003

Dorr, G.J., Hewitt, A.J., Adkins, S.W., Hanan, J., Zhang, H., Noller, B., 2013. A comparison of initial spray characteristics produced by agricultural nozzles. Crop Prot. 53, 109–117. https://doi.org/10.1016/j.cropro.2013.06.017

Draxler, R.R., 1976. Determination of atmospheric diffusion parameters. Atmos. Environ. 10, 99–105. https://doi.org/10.1016/0004-6981(76)90226-2

Holterman, H.J., 2003. Kinetics and evaporation of water drops in air, IMAG rapport;2003-12. https://doi.org/10.18174/562300

Jones, F.E., Harris, G.L., 1992. ITS-90 density of water formulation for volumetric standards calibration. J. Res. Natl. Inst. Stand. Technol. 97, 335–340. https://doi.org/10.6028/jres.097.013

Kim, Y.J., Kim, Y.-J., Han, J.-G., 2004. Numerical analysis of flow characteristics of an atmospheric plasma torch 1–8.

Lebeau, F., Verstraete, A., Stainier, C., Destain, M.-F., 2011. RTDrift: A real time model for estimating spray drift from ground applications. Comput. Electron. Agric. 77, 161–174. https://doi.org/10.1016/j.compag.2011.04.009

Løfstrøm, P., Bruus, M., Andersen, H.V., Kjær, C., Nuyttens, D., Astrup, P., 2013. The OML-SprayDrift model for predicting pesticide drift and deposition from ground boom sprayers. J. Pestic. Sci. 38, 129–138. https://doi.org/10.1584/jpestics.D12-064

Mackay, D., Wesenbeeck, I. van, 2014. Correlation of Chemical Evaporation Rate with Vapor Pressure. Environ. Sci. Technol. 48, 10259–10263. https://doi.org/10.1021/es5029074

Souto, M.J., Souto, J.A., Pérez-Muñuzuri, V., Casares, J.J., Bermúdez, J.L., 2001. A comparison of operational Lagrangian particle and adaptive puff models for plume dispersion forecasting. Atmos. Environ. 35, 2349–2360. https://doi.org/10.1016/S1352-2310(00)00537-9

Stull, R., 2011. Wet-bulb temperature from relative humidity and air temperature. J. Appl. Meteorol. Climatol. 50, 2267–2269. https://doi.org/10.1175/JAMC-D-11-0143.1

Sypka, P., Starzak, R., 2013. Simplified, empirical model of wind speed profile under canopy of Istebna spruce stand in mountain valley. Agric. For. Meteorol. 171–172, 220–233. https://doi.org/10.1016/j.agrformet.2012.11.003

Zannetti, P., 1990. Gaussian Models, in: Air Pollution Modeling. Springer US, Boston, MA, pp. 141–183. https://doi.org/10.1007/978-1-4757-4465-1_7